## PROJECT   REPORT

## Computational  Modelling  Of  Dendrite  Formation

Submitted By

Prashanth  C N

(Roll no. 135135)

B.Tech  lll  year

Department of Metallurgical and Materials Engineering

National Institute of Technology ,Warangal

Warangal,  Telangana – 506004

Under the guidance of

Prof. T. A. Abinandanan

Department of Materials Engineering

Indian Institute of Science, Bengaluru -560012

## COMPUTATIONAL  MODELING  OF  DENDRITE  FORMATION

## ABSTRACT

A web app is developed that runs simulations of the following models on a web browser using client-side scripting.

Diffusion equation in 1D and in 2D

Phase field equation in 1D and in 2D (isotropic)

Phase field equation in 2D (anisotropic).

## INTRODUCTION

Crystal growth is one of the most fascinating phenomenon of spontaneous pattern formations in nature. Dendritic structure is commonly seen in solidification of metals or crystallization in supersaturated solution.  In the current context, we are interested in metallic dendrites formed when a metal or an alloy of multiple metals, in liquid form freezes. A dendrite is a crystal with a tree-like branching structure. The success of these simulations is determined by the degree to which the results obtained from these simulations correctly reflect the growth of actual dendrites. Here we simulate in 1 and 2 dimensions.

Here all the equations were solved using Finite Difference Method. Taylor’s Series has been used for the deduction of differential terms, so while choosing dx and dt values one has to make sure that these values satisfy Neumann criterion.

dt(dx)2   ≤ 12D

If Neumann criterion is not satisfied, then results are affected and desired results are not obtained.

# PHYSICAL  DERIVATIONS

## Heat Equation  in 1D

In a metal rod with non-uniform temperature, heat (thermal energy) is transferred from regions of higher temperature to regions of lower temperature. Three physical principles are used here.

- Heat (or thermal) energy of a body with uniform properties:

Heat energy = c m u ,

where m is the body mass, u is the temperature, c is the specific heat, units[c]= L2T−2U−1 (basic units are M mass, L length, T time, U temperature). c is the energy required to raise a unit mass of the substance 1 unit in temperature.

- Fourier’s law of heat transfer: rate of heat transfer proportional to negative temperature gradient,

Rate of heat transferarea= -K0∂u∂x

where K0 is the thermal conductivity, units [K0]= MLT−3U−1 .In other words, heat is transferred from areas of high temperature to low temperature.

- Conservation of energy:

Consider a uniform rod of length l with non-uniform temperature lying on the x-axis from x=0 to x = l. By uniform rod, it is meant that the density ρ, specific heat c, thermal conductivity K0, cross-sectional area A are all constant. Assume the sides of the rod are insulated and only the ends may be exposed. Also assume there is no heat source within the rod. Consider an arbitrary thin slice of the rod of width Δx between x and x+Δx. The slice is so thin that the temperature throughout the slice is u (x, t). Thus,

Heat energy of segment = c × ρAΔx × u = cρAΔxu(x, t)

By conservation of energy,

_

Heat out from right boundary

Heat in from left boundary

=

Change of heat energy of segment in time Δt

From Fourier’s Law (1),

cρA∆xux,t+∆t- cρA∆xux,t=∆tA-K0∂u∂xx- ∆tA-K0∂u∂xx+∆x

Rearranging yields,

ux, t+∆t-u(x,t)∆t=K0cρ∂u∂xx+∆x-∂u∂xx∆x

Taking the limit ∆t, ∆x→0 gives the Heat Equation,

∂u∂t= κ∂2u∂x2

where

κ= K0cρ

is called the thermal diffusivity, units κ = L2 / T.

### Initial and Boundary Conditions

To make use of the Heat Equation, we need more information:

1. Initial Condition (IC): in this case, the initial temperature distribution in the rod u(x,0).

2. Boundary Conditions (BC): in this case, the temperature of the rod is affected by what happens at the ends, x =0, l. what happens to the temperature at the end of the rod must be specified. In reality, the BCs can be complicated. The boundary conditions considered in the web application has been described in details later.

## Heat Equation 2D

The derivation above can be extended similarly for a general case, not limited to 1D. In this case, no assumption regarding the insulation of the walls of the rod has to be made. The result of derivation obtained is

∂u∂t= κ∇2u

In 2-D the above equation takes the following form:

∂u∂t= κ∂2u∂x2+∂2u∂y2

## Phase field model

A number of physical phenomena involve motion of boundaries between entities which are homogeneous with respect to certain physical properties. Examples of such phenomenon are solidification, precipitation, phase-separation, order-disorder transitions etc. A notable feature of such problems is that these involve mass, heat or momentum transfer across interfaces which are the boundaries between physically homogeneous media (phases). Such problems can be modeled quite elegantly using diffuse-interface models.

This model allows discontinuity in a physical property across the interface to vary over a finite width. The order parameter distinguishes between the phases and is chosen among the various extensive variables distinguishing the phases.

The model uses an Allen-Cahn type model coupled with a diffusion equation. For this, let us assume the example of pure material solidification, wherein due to heat diffusion, the melt gets cooled leading to solidification. So, in principle we are going to need two evolution equations, one for the field: order-parameter φ whose evolution will indicate phase transformation from liquid (φ =0) to φ =1 that is the solid; the second, will be the evolution of the temperature field T.

The evolution equation for temperature field can be derived from conservation of internal energy as:

∂e∂t= -∇.J,

= ∇. (K∇T)

Using Fourier’s law, such that J= -K∇T. e is the internal energy of the system and can be written as an interpolation of internal energies of the solides and the liquidel, using order parameterφ.

es=CvT

el=CvT+L

Cv is the specific heat, and for simplification it is considered same for the solid and the liquid, and L is the latent heat. Using interpolation function,

hφ= φ2(3-2φ)

the internal energy takes the form as:

e=eshφ+elh1-φ

Using Allen-Cahn equation in the above equation along with the evolution equation we get,

τϵ∂φ∂t=2γϵ∂2φ∂x2-18γφ ϵ1-φ1-2φ- LT-TMTM6φ1-φ

Where ϵ is the interface width, γ is the surface energy, τ is a small positive constant and L is the latent heat.

Note: LT-TMTM is the driving force for the movement of interface, when the temperature is below TM, there is a positive evolution of the variable φ implying solidification. Similarly, the reverse would indicate melting, i.e. T > TM.

And, the evolution temperature for temperature field takes the form,

ρC∂T∂t=γ∂2T∂r2+6Lφ1-φ∂φ∂t

### Initial and Boundary Conditions

To make use of the above set of evolution equations, we need more information:

1. Initial Condition (IC): in this case, undercooling temperature is required as the complete system is at this temperature in the beginning, and an initial order parameter profile which indicates the amount of solid present and its position.

2. Boundary Conditions (BC): in this case, the temperature and order parameter of the system are affected by what happens at the ends/boundaries. In reality, the BCs can be complicated. The boundary conditions considered in the web application has been described in details later.

# MODELS  &  SIMULATIONS

## 1. Diffusion Equation in 1D

Variable:  Tx,t                                x = distance from left end

t = time

∂2T∂x2 = 1D ∂T∂t

D = kρC

Where

k = Thermal diffusivity

ρ = Mass density

C = Specific heat

D = Diffusion Coefficient

On opening the web app, this is how the page looks (for Diffusion 1D)

The following fig. shows the simulation

In the above page one can vary dx, dt, diffusion coefficient, and number of points in Control Panel. Simulation can be seen on the left part of the page.

Randomly distributed Temperature

Smoothening of sharp points

About to reach steady state

Steady state (Same temperature at all points)

Maximum Temperature					Minimum Temperature

Initially, there is a random distribution of temperature. Red color corresponds to maximum temperature blue color to minimum temperature and the colors in between red and blue represents the temperature in between maximum and minimum temperature. It is assumed that the upper and lower edges are thermally insulated i.e. no heat transfer takes place through top and bottom edge.

Once the simulation starts diffusion takes place and the sharpness of the color decreases. Finally, at steady state (at infinite time) all the points will have same temperature. And the corresponding color will be somewhere in between red and blue (not necessarily the mid way between the two). In theory it will take infinite time to attain steady state. One can also get the value of temperature at any point and at any instant by hovering the cursor on that point.

There are two boundary conditions: Dirichlet Boundary Condition and Neumann Boundary Condition. A Dirichlet Boundary Condition means imposing the value for the temperature in edge of the domain, i.e. fixing the value for the temperature for the solution at that edge while Neumann Boundary Condition mean to impose the flux of heat through that edge. To keep it simple, edge temperature in Dirichlet Boundary Condition is taken as the average of maximum and minimum temperature and the flux of heat through that edge as zero in Neumann Boundary Condition.

## 2. Diffusion equation in 2D

Variable :  Tx,y,t                            x = distance from left end

Y = distance from top end

t = time

∂2T∂x2  +  ∂2T∂x2  = 1D ∂T∂t

D = kρC

Where

k = Thermal diffusivity

ρ = Mass density

C = Specific heat

D = Diffusion Coefficient

The simulation of the model is as shown below.

Randomly  distributed  temperature

Smoothening  of  sharp  points

About  to  attain  steady  state

Steady  state

This is very similar to the previous model. The only difference is that in previous case we have assumed top and bottom edges to be thermally insulated but here the conditions of these edges are similar to the other two edges. Boundary Conditions are applied on all boundaries (all 4 edges).

## 3. Phase Transformation Equation in 1D  (isotropic)

With phase transformations, normally at least one new phase is formed that has different physical/chemical characteristics and/or a different structure than the parent phase. This model includes two variables; one is a phase field ρ(x,t) and the other is a temperature field  T(x,t). The variable ρ(x,t) is an ordering parameter at the position x and the time t , ρ=0  means being liquid and ρ=1 solid. And the liquid solid interface is expressed by the steep layer of ρ connecting the values 0 and 1.

τϵ∂φ∂t  =  2γϵ∂2φ∂x2 -18γφ ϵ1-φ1-2φ- LT-TMTM6φ1-φ

And

ρC∂T∂t =  γ∂2T∂x2  +  6Lφ1-φ∂φ∂t

Where

T = temperature

C = Heat Capacity

γ = Surface Energy

x = distance from left end

t = time

L = Latent Heat

τ = a small positive constant

ϵ = width of the interface

Here we concentrate on the growth of pure material. Initially the vessel is filled with a molten metal of undercooled temperature, and along the right wall the solid exists. The vessel is cooled from right end. Solidification occurs from the right end and the crystal grows towards left end. As the molten metal solidifies, there is a release of latent heat and the temperature near the interface. This latent heat is taken away through the solid phase to the right. The shape of the interface becomes flat very quickly. The interface never becomes complicated and it acquires a shape with higher symmetry. This is because not only the surface tension but also the temperature field make the shape smooth and simple.

In the above page one can vary dx, dt, thermal conductivity, surface energy, heat capacity and interface width in Control Panel. Simulation can be seen on the left part of the page. First graph gives variation of φ  and the second temperature with time and distance from left end. Initially, temperature is constant and interface width is very small as there is a sharp change from solid phase to liquid phase.

When simulation starts temperature at interface increases and interface width increases by diffusion of liquid and solid phases. Interface moves towards left end and the heat near interface diffuses. As time proceeds temperature of the vessel becomes constant and no more solidification takes place thereafter.

Variation of order parameter ( φ) with time

Liquid 									Solid

x (distance from left end)

Variation  of temperature (T) with time

x (distance from left end)

## 4. Phase Transformation Equation in 2D  (isotropic)

Most of the phase transformations do not occur instantaneously. Rather, they begin by the formation of numerous small particles of the new phase. Phase transformation involves two stages : nucleation and growth. Growth takes place after the nucleus reaches a critical size. If not the embryo formed dissolves back into the solution. In this model Critical Radius = 10. So, if the radius of the nucleus is set to less than 10, it dissolves giving complete liquid else nucleus grows to give a solid crystal.

τϵ∂φ∂t =2γϵ(∂2φ∂x2 +∂2φ∂y2 )-18γφ ϵ1-φ1-2φ-LT-TMTM6φ1-φ

And

ρC∂T∂t =  γ(∂2T∂x2 +∂2T∂y2 ) +  6Lφ1-φ∂φ∂t

Where

T = temperature

C = Heat Capacity

γ = Surface Energy

r = radial distance

t = time

L = Latent Heat

τ = a small positive constant

ϵ = width of the interface

The simulation is shown below for the nucleus having radius greater than the critical radius ( here it is 10). Growth of the nucleus can be seen here.

Growing solid nucleus             				Temperature  profile

This is similar to the previous model. Here the crystal grows radially but not in the previous case. The vessel with molten metal is undercooled and nucleus is formed .  The shape of the interface is rounded as it moves and disappears, in the end.

## 5. Phase Transformation Equation in 2D  (anisotropic)

τϵ∂φ∂t =2γϵ(∂2φ∂x2 +∂2φ∂y2 )-18γφ ϵ1-φ1-2φ-LT-TMTM6φ1-φ

And

ρC∂T∂t =  γ(∂2T∂x2 +∂2T∂y2 ) +  6Lφ1-φ∂φ∂t

Dendrites are formed when surface anisotropy is included in the system, which means that there are going to be some preferred directions for solidification.

Anisotropy can be introduced by assuming that γ depends on the direction of the outer normal vector at the interface. So γ is represented as a function of  θ as

γ = γo (1 + δ cos4θ )

Where ,

δ gives the strength of anisotropy.

θ is the angle made by the vector directing radially outwards at the point under           consideration with horizontal.

It is a four fold anisotropy  where  4  dendrites are formed.

Cos θ = φxφx2+φy2

Cos 4θ = 4(φx4+φy4)(φx2+φy2)2 -3

The simplest finite difference scheme with a nine point laplacian is used to solve equations . First, the new value of phase field variable is calculated to substitute it into the temperature field to get the new value of temperature field. Nine point laplacian is required for the stability of the solution. The growth of dendrite takes place as shown below.

## CONCLUSION

Two-dimensional dendritic behaviour has been reproduced by numerically integrating the time dependent thermal diffusion/phase change problem. The symmetry is controlled by expressions of anisotropy. If this effect can be sufficiently reduced by further development, a significant step in numerical microstructure simulation will be reached.